RC Filter Networks

Lennon O Náraigh, 01020021

3rd April 2003

Abstract:

This experiment aims to examine the response of R-C filter networks to sinusoidal signals of various frequencies, and to various square waves.  In particular, the high-pass and low-pass circuits will be investigated.

Basic Theory and Equations:

The high-pass filter consists of a capacitor and resistor in series.  We measure an output voltage, Vout and an input voltage, Vin.  Now from figure 1 we see that Vin is divided between the capacitor C and the resistor R.  To find an expression for the ratio VoutVin, we proceed as follows.

For any component in an alternating current (a.c.) circuit, across which there is a varying voltage with maximal value V0, and a current with maximal value I0, define the reactance of that component, by X≡V0I0.  Thus, for the resistor of figure 1, XR=R.  Now suppose that a sinusoidal current given by i(t)=I0cosωtflows in the capacitor, the voltage across the capacitor is as follows:

C=dqdvCdvdt=dqdt=i(t)=I0cosωtI0Ccosωtdt=dvv(t)=I0Cωsinωt=V0sinωt=Vcosωt−π2

Hence, XC=1Cω								(1)

We see that vR=iRR=RI0ejωt and that vC=iCXC=1CωI0ejωt−π2, where, by writing the voltages in complex form we take the real part of the exponential.  This shows that vR leads vC by π2, and that the phase difference between the current and the total voltage (i.e. the phase difference between the voltage across the resistor and the total voltage) can be obtained by adding vC and vR in the complex plane:

Figure 2

tanφ=VCVR=XCR=12πRC1f						(2)

Furthermore, this diagram enables us to find the net input voltage:

V2=I02R2+I02XC2=I02R2+XC2≡Vin2Vout2=I02R2VoutVin2=R2R2+1ωC2=11+1ωRC2					(3)

Taking logs on both sides, we have the following:

log10VoutVin=12log1011+12πfRC2					(4)

We see that for small f, 11+12πfRC2 is small, so the output voltage is small.  Similarly, for large f, 11+12πfRC2 is close to unity, so the output equals the input.  Thus, such a configuration is called a high-pass filter.

For the low-pass filter (figure 3), the output voltage is that across the capacitor.  Clearly,

VoutVin2=XC2XC2+R2=11+ωRC2						(5)

For large  (i.e. large f), VoutVin is nearly zero.   Similarly, for small  (i.e. small f), VoutVin is nearly unity.  This is why such a configuration is called a low-pass filter.

Also, the equation for the phase difference , between the input and output voltages, is the same as before.

Finally, we use a square wave as a voltage source and observe the outcome.  For each half-cycle the R-C circuit behaves like a simple direct current (d.c.) circuit, because the input voltage is equal to a constant voltage, given by .  Consequently, the circuit behaves like a simple d.c. R-C circuit, and the equations for a charging capacitor apply for the current, hence the voltage, across the capacitor:

q(t)=CE1−e−tRC≡CE1−e−tτv=qCv=E1−e−tτE=vin

vout=vin1−e−tτ									(6)

Method and Results:

- High-Pass Filter

The aim of this experiment is to verify equations (2) and (3) and to find the so-called “half power point” – i.e. the value of f for which VoutVin2=12.  From equations (2) and (3) it should be clear that the value of  for which this happens is ±π4.  The apparatus was set up as in figure 3 below, and the quantities Vout and f were measured, for fixed Vin, over a large range of frequencies.  The following data were obtained:

| f / Hertz | VoutmV |  | VinmV |  | VoutVin | δVoutVin |  | δlog10VoutVin |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 100 | 12.6 |  | 84.8 |  | 0.149 | 0.007 | -0.827 | 0.020 |
| 1000 | 61.2 |  | 89.6 |  | 0.683 | 0.009 | -0.166 | 0.006 |
| 2000 | 76.0 |  | 85.6 |  | 0.888 | 0.011 | -0.052 | 0.005 |
| 5000 | 82.4 |  | 86.4 |  | 0.953 | 0.015 | -0.020 | 0.007 |
| 8000 | 84.8 |  | 86.4 |  | 0.981 | 0.011 | -0.008 | 0.005 |
| 10000 | 86.8 |  | 88.8 |  | 0.978 | 0.011 | -0.010 | 0.005 |
| 20000 | 86.4 |  | 88.0 |  | 0.981 | 0.011 | -0.008 | 0.005 |
| 50000 | 92.0 |  | 89.6 |  | 1.027 | 0.011 | +0.011 | 0.005 |
| 100000 | 102 |  | 105 |  | 0.971 | 0.010 | -0.013 | 0.004 |
| 500000 | 260 |  | 264 |  | 0.984 | 0.004 | -0.007 | 0.002 |

δVout=0.5mV

δVin=0.5mV

δVoutVin=VoutVinδVoutVout+δVinVinδlog10VoutVin=1log10δVoutVinVoutVin=0.434δVoutVinVoutVin=0.434δVoutVout+δVinVin

Figure 4:

| f / Hertz | Δtμs | δΔtμs | φ≡2πfΔt | δφ≡2πfδΔt | tanφ | δtanφ |
| --- | --- | --- | --- | --- | --- | --- |
| 100 | 244 | 5 | 0.153 | 0.003 | 0.154 | 0.003 |
| 1500 | 67.6 | 0.5 | 0.637 | 0.004 | 0.740 | 0.006 |
| 1600 | 60.8 | 0.5 | 0.611 | 0.005 | 0.700 | 0.007 |
| 1700 | 54.8 | 0.5 | 0.585 | 0.005 | 0.663 | 0.007 |
| 2000 | 40.4 | 0.5 | 0.508 | 0.006 | 0.557 | 0.006 |
| 8000 | 2.8 | 0.5 | 0.140 | 0.025 | 0.142 | 0.025 |
| 10000 | 1.0 | 0.5 | 0.063 | 0.031 | 0.063 | 0.031 |
| 20000 | 0.5 | 0.05 | 0.063 | 0.006 | 0.063 | 0.006 |
| 50000 | 0.12 | 0.05 | 0.038 | 0.016 | 0.038 | 0.016 |
| 100000 | 0.06 | 0.05 | 0.038 | 0.031 | 0.038 | 0.031 |
| 500000 | 0.006 | 0.0005 | 0.002 | 0.000 | 0.002 | 0.000 |

δtanφ=1cos2φδφ

Figure 5:

Slope≡m=12πRC=1111.4±24.1

This graph verifies the linear relationship implied by equation (2).

Figure 6

By inspection of figure 4, the half-power point (the value of f for which VoutVin=12) occurs for log10f=3.  This is clearly why this is called the 3 dB point.  By the linear equation accompanying figure 5, the value of tan for which this occurs is 1.11±0.03  Thus, , where δφHPP=1tanφHPP2+1δtanφHPP. The actual value is π4=0.785, which agrees with the measured value, to within experimental error.  Using a logarithmic scale enables one to compare a large range of frequencies while considering graphs that only involve a small numerical interval.

- The low-pass filter:

Instead of regarding the output voltage, Vout to be across the resistor, the output voltage is considered to be that across the capacitor.  The circuit was set up as in figure 7, with the appropriate values for R and for C being used.

The half power-point and the phase shift at this frequency were measured, and this was compared with the theory.

f1=1.106±0.0005kHzf2=1.098±0.0005kHzf≡f1+f22=1.102±0.0004kHz=1102HzΔt=108.0±0.5μsφ=2πfΔt=2π1.102±0.00004kHz×108.0±0.5μs=0.748±0.003

The half-power point is obtained from theory as follows:

VoutVin2=12=11+ωRC2⇒f=12π1RC

Using the values for R and C in figure 7, and taking a 5% tolearance for the resistor and capacitor (as indicated on these devices), we get

f=1064±106.4Hz, which agrees with the measured value, to within experimental error.  The measured value of  at this frequency differs from the theoretical value π4=0.785 by an amount not explained by the uncertainty.  However, the uncertainty given is very small.

- Square wave response of filter circuits

A square wave was applied to both the high pass and low pass circuits.  Three frequencies were applied, for which T=1f<<RC, T=1f~2RCand T=1f>>RC.  In the first case, the frequency of the input voltage is much greater than 1τ≡1RC, where τis the time constant of the circuit.  Thus, for large frequencies, the effect of the capacitor charging during each half-cycle is not observable.  On the other hand, for f~12τ, i.e. T~2τ, the effect of the discharging capacitor is observable, and the time constant can be measured.

The high-pass filter:

For f>>1RC=10.022×10−6F6.8×103Ω=6684Hz(large f), the output voltage is nearly equal to the input voltage, and the effects of the charging capacitor are not observed.

Insert sketch

For f~12τ≈13,000Hz, we can see the capacitor charging and discharging, each half cycle.

Insert sketch

The value of t for which voutvin=1−1e=0.63(from equation 6) is the time constant.

Insert sketch

By inspection of the graph, we find that τ=180±20μs, which compares with the value, as read from the resistor and capacitor, of 150±14μs.  We note that these agree, to within experimental error.

For f<<1RC, the output voltage is nearly zero, and the only point in the waveform of vout for which vout is non-zero is where the capacitor charges.

Insert sketch

The low-pass filter:

We have the following results for the low-pass filter:

Forf>>1RC=10.22×10−6F680×103Ω=6684Hz,vout is nearly zero, and the only point in the waveform of vout for which vout is non-zero is where the capacitor charges.

For f~12τ≈13,000Hz, we can see the capacitor charging and discharging, each half cycle.

By inspection of the graph of the charging capacitor (vout as a function of time), we find that τ=160±20μs, which compares with the value, as read from the resistor and capacitor, of 150±14μs.  We note that these agree, to within experimental error.

For f<<1RC, the output voltage is nearly equal to the input voltage, and the effects of the charging capacitor are not observed.

From the equations

i(t)=I0cosωtvR=I0RcosωtdvC=dqCi(t)=dvCdt=vRR
it follows that vC=1RCtvRdt and that vR=RCdvCdt.

Now in the case of the high-pass circuit, it is the voltage across the resistor which we observe.  We see that this is equal to RC times the derivative of the voltage across the capacitor.  This is why, in this context, the high pass filter is called a differentiating circuit.  Similarly, in the low-pass circuit, it is the voltage across the capacitor which we observe, and this is given by 1RC times the integral of the voltage across the resistor.  Thus, the low-pass filter is considered to be an integrating circuit.

Conclusion:

The frequency corresponding to the half-power point of the high-pass filter was found to be at f = 1000Hz, while the phase difference at this frequency was found to be φHPP=0.84±0.10.  These values agree with the predicted values, to within experimental error.

The frequency corresponding to the half-power point of the low-pass filter was found to be at f = 1102Hz, while the phase difference at this frequency was found to be φHPP=0.748±0.003.  While the latter result does not agree with the theory, the experimental error is very small.

The response of the two filters to square waves was noted, and from this, the time constants of the two systems were found.  In the case of the high-pass filter, this was found to be equal to τ=180±20μs, while in the second case – that of the low-pass filter, the time constant was found to be equal to τ=160±20μs.  The predicted value – in both cases – is τ=150±14μs, so these values agree, to within experimental error.

Finally, it was noted that the high-pass filter performs the function of a differentiating circuit, while the low-pass filter performs the function of an integrating circuit.